SYNAPTIC FABRIC

Nexus Edition · Project Report

Autonomous Industrial AI · Predictive Maintenance Platform

Project Report Autonomous Industrial AI · Predictive Maintenance Platform Repository: `Siddhi-Karhekar/SynapticFabricfinal`

## Table of Contents

1. Executive Summary

2. System Architecture

3. The Manufacturing Chain

4. The AI Trinity

5. Digital Twin Simulator

6. Backend Service (FastAPI)

7. Closed-Loop n8n Agent Pipeline

8. Local Two-Tier Chatbot

9. Maintenance Logging & Audit Trail

10. What-If Spike Scenarios

11. Auto-Maintenance Logic

12. Frontend Dashboard

13. Data Model & Persistence

14. Deployment & Local Setup

15. Operational Gotchas

16. Endpoint Reference

17. WebSocket Payload Reference

18. Tuning History (What Changed and Why)

19. Repository Structure

20. Future Work & Known Limitations

## 1. Executive Summary

Synaptic Fabric is a fully local, autonomous IIoT predictive-maintenance platform that simulates a 4-node manufacturing chain in real time and uses a triad of AI models (a Physics-Informed Neural Network, a chain-aware Graph Neural Network, and classical ML predictors) to forecast failures, justify them with Retrieval-Augmented Generation (RAG), and self-correct via a local n8n agent loop — all without any cloud dependency.

The system is built around three principles:

1. Physics first. Every cross-machine effect is modelled with explicit causal-delay queues, not random noise. The PINN's loss is a heat-equation residual.

2. Local-only. n8n (Community Edition), Qdrant, and Ollama (phi3 / phi3:mini) run in Docker on the host. No external API calls.

3. Closed loop. When a machine drifts into Critical, FastAPI fires a webhook into n8n; n8n routes on severity, asks phi3 to justify the action, and posts a corrective AUTO_MAINTENANCE back to FastAPI which resets the digital-twin state — visible in the dashboard within seconds.

A React dashboard visualizes the four machines as both individual cards and a 3D digital twin, with collapsible panels for maintenance history, exports, and live temperature charts. An inline what-if spike feature lets operators perturb any machine's state and see the chain-effect ripple to the rest of the plant — fully reversible by any subsequent maintenance action.

The platform is engineered to demonstrate three things at once: applied AI (PINN + GNN + ML fusion + SHAP explainability), agentic systems (n8n-driven closed loop with cited-LLM reasoning), and operational realism (durable audit logs, what-if simulations, soft-failure handling when components like Ollama or n8n are offline).

## 2. System Architecture

┌────────────────────────────────────────────────────────────────────────┐

│  HOST                                                                  │

│                                                                        │

│  ┌─ FastAPI (uvicorn :8000) ────────────────────────────────────────┐  │

│  │   • WS /ws/machines     — 1 Hz live stream                       │  │

│  │   • Digital-twin tick   — physics propagation + 4 AI fusions     │  │

│  │   • DB writes           — MachineLog + MaintenanceLog (SQLite)   │  │

│  │   • REST endpoints      — /history, /maintenance, /spike, /chat  │  │

│  │   • n8n outbound        — fire-and-forget alerts on Warning+     │  │

│  │   • n8n inbound         — POST /agent/action  (corrective ops)   │  │

│  └──────────────────────────────────────────────────────────────────┘  │

│            ▲                                                           │

│            │ WebSocket                                                 │

│  ┌─ React CRA (npm start :3000) ────────────────────────────────────┐  │

│  │   • App.js / MachineCard / FactoryTwin3D / Chatbot               │  │

│  │   • useMachineStream hook (popup queue, reconnect)               │  │

│  │   • Inter + JetBrains Mono · industrial control-room aesthetic   │  │

│  └──────────────────────────────────────────────────────────────────┘  │

│                                                                        │

└──────────────────────────────────────┬─────────────────────────────────┘

│ docker network (host-mapped)

▼

┌──────────────────────────────────────────┐

│  DOCKER (docker-compose.yml)             │

│                                          │

│   ┌─ n8n        :5678  ───────┐          │

│   │ Synaptic Fabric workflow  │          │

│   │ Webhook → Switch → Ollama │          │

│   │                  → REST   │          │

│   └───────────────────────────┘          │

│                                          │

│   ┌─ Qdrant     :6333  ─┐                │

│   │ Vector DB (RAG)     │                │

│   └─────────────────────┘                │

│                                          │

│   ┌─ Ollama     :11434 ─┐                │

│   │ phi3, phi3:mini      │               │

│   └─────────────────────┘                │

└──────────────────────────────────────────┘

### Why these boundaries

- Host runs the realtime services (FastAPI + React) because:

- The simulator is CPU-bound and needs predictable scheduling.

- The frontend dev loop is faster outside containers.

- *Docker holds the infrastructure*** (n8n, Qdrant, Ollama) because:

- They have stateful volumes that benefit from named-volume persistence.

- Each upgrades on its own cadence and would otherwise pollute the host venv.

- n8n is intentionally NOT in the chat path. The dashboard chatbot's /chat/stream is a single-process two-tier router (regex fast path + slim LLM path); routing through n8n would add HTTP-hop latency to every keystroke.

## 3. The Manufacturing Chain

### Topology

M_1 Induction Motor  →  M_2 Industrial Gearbox  →

M_3 CNC Milling Tool →  M_4 Robotic Sorting Arm

Each machine runs as a stateful entry in digital_twin/simulator.py::MACHINE_MEMORY:

| Field | Range | Notes |
| --- | --- | --- |
| temperature | 290 – 330 K | Clamped each tick |
| torque | 30 – 95 Nm | Wear-coupled + spike noise |
| tool_wear | 0 – 1 | Monotonically increasing in steady state |
| vibration_index | 0 – 1 | Self-decaying with upstream injection |

### Causal propagation

torque spike (M_1) → vibration (M_2) → thermal runaway (M_3) → overstrain (M_4)

Implemented with explicit delay queues (PROP_BUFFERS), not RNG. Each edge holds delay ticks of upstream values; what comes out is what went in delay seconds ago:

PROPAGATION_DELAY = {

"M_1->M_2": 2,   # torque spike reaches gearbox after ~2 ticks

"M_2->M_3": 3,   # vibration drives milling thermal runaway

"M_3->M_4": 4,   # thermal stress shows up as arm overstrain

}

This is what makes upstream → downstream effects measurably delayed in a way that shows up correctly in the dashboard charts (vibration spikes always lead temperature rises in M_3, etc.) and gives the GNN something to learn.

## 4. The AI Trinity

### 4.1 Physics-Informed Neural Network (PINN)

Location: `pinn_model/heat_pinn.py`, inference in `pinn_model/pinn_inference.py`.

Predicts the steady-state temperature a machine should be at, given its current thermal load and ambient air. The loss function is the residual of the 1-D heat equation, not a curve-fit on temperature data — the network learns the underlying physics.

The fused predictor uses the gap between PINN output and measured temperature:

pinn_delta = max(0.0, pinn_temp - temperature)

pinn_risk = min(pinn_delta / 15.0, 1.0)

If the heat equation says the machine will be hot but it's currently cooler, that's an early indicator of developing thermal runaway and bumps risk. This is the explainable, physics-grounded leg of the trinity.

### 4.2 Chain-aware Graph Neural Network (GNN)

Location: `ml_models/gnn_model.py`, `ml_models/gnn_inference.py`, `ml_models/graph_builder.py`.

Encodes the manufacturing chain as a directed graph with self-loops + weighted reverse edges (current weights):

adj[i][i]    = 1.0    # self-loop

adj[i+1][i]  = 0.5    # downstream receives upstream

adj[i][i+1]  = 0.15   # weak reverse for attribution

# adj is then row-normalized so message passing is bounded.

After row normalization, each row's columns sum to 1 — node i's GNN feature is a convex combination of its own state, its predecessor, and a small reverse echo from its successor.

The pretrained checkpoint at ml_models/gnn.pth was trained on raw unscaled features (temperature ~300, torque ~50). Its single Linear(4,1) produced near-constant logits that sigmoided to ~1.0 across all machines — a constant 0.15 floor on every fused prediction. The current implementation bypasses the saturating model entirely and replaces it with a chain-aware physics fallback:

per_machine_risk = bounded([

(T-290)/50 * 0.3 + tool_wear * 0.3 + vibration * 0.3 + (torque/100) * 0.1

for each machine

])

chain_risk = adj @ per_machine_risk      # row-normalized message pass

This preserves the chain-aware semantic but produces sensible varying numbers (e.g. M_3 GNN risk ≈ 0.18 ± 0.05 instead of 1.000).

### 4.3 Classical ML stack

| Model | File | Role |
| --- | --- | --- |
| Failure-probability model | ml_models/failure_model.py | Per-machine binary risk classifier (trained on AI4I 2020 dataset) |
| Anomaly detector (ML) | ml_models/anomaly_model.py | Unsupervised anomaly score |
| Transformer (short-horizon) | ml_models/transformer_inference.py | Future-temperature forecast over a 5–10 tick window |
| SHAP explainer | ml_models/explainer.py | Per-feature attribution for the failure predictor |

### 4.4 Fusion

The final per-machine prediction in backend_fastapi/ai_engine/machine_analyzer.py:

prediction = (

0.40 * failure_probability +   # ML

0.20 * anomaly_score +         # physics + ML anomaly merged

0.15 * gnn_risk +              # chain-aware

0.15 * pinn_risk +             # heat-equation deviation

0.10 * ml_anomaly              # ML anomaly model

)

# Hard physics override

if tool_wear > 0.9 or temperature > 315 or vibration > 0.85:

prediction = max(prediction, 0.9)

elif tool_wear > 0.75 or temperature > 305 or vibration > 0.65:

prediction = max(prediction, 0.7)

Health bins: ≥ 0.85 → Critical, ≥ 0.5 → Warning, else Healthy.

## 5. Digital Twin Simulator

Location: `digital_twin/simulator.py`.

### Per-machine dynamics (one tick = ~1 second)

| Machine | Drives | Notes |
| --- | --- | --- |
| M_1 Induction Motor | Torque baseline + 4%-probability spikes (torque += U(8, 18)). Tool wear gain ~0.0005/tick. Temperature climbs slowly; bigger jump if torque > 70 (I²R heating). |  |
| M_2 Industrial Gearbox | Receives delayed_torque via M_1→M_2 queue. Vibration: 0.985 · V_old + 0.0003 + 0.012 · delayed_torque + noise. |  |
| M_3 CNC Milling Tool | Receives delayed_vib via M_2→M_3 queue. Vibration: 0.94 · V_old + 0.18 · delayed_vib. Heat: friction_heat * 0.4 + vib_heat * 0.15. |  |
| M_4 Robotic Sorting Arm | Receives delayed_heat via M_3→M_4 queue. Vibration & torque coupled to upstream heat. |  |

### Shared mechanisms

- Natural cooling above 300 K.

- Damage-driven heat amplification when tool_wear > 0.6 (and stronger above 0.85).

- Hard clamps on every field at the end of each tick.

- reset_inbound_buffers(machine_id) zeroes every PROP_BUFFER queue feeding into a maintained machine — without this, maintenance resets the machine state but the queued upstream disturbances re-inject the same pre-maintenance values within 3 ticks, snapping the machine straight back into Critical.

### Causal-delay buffers

PROP_BUFFERS = {

edge: deque([0.0] * delay, maxlen=delay)

for edge, delay in PROPAGATION_DELAY.items()

}

Push-and-pop semantics: _push_propagation(edge, value) appends value and returns the oldest value in the deque. Combined with maxlen, this gives an exact delay-tick FIFO with no extra clock-keeping logic.

## 6. Backend Service (FastAPI)

Location: `backend_fastapi/app/main.py`

### Responsibilities (per WS tick)

1. Step the digital twin → 4 machine dicts.

2. Apply maintenance cooldown decay.

3. Run AI fusion via machine_analyzer.analyze_machines().

4. Persist a MachineLog snapshot (SQLite).

5. Periodically (every 60 s) clean out MachineLog rows older than 30 minutes.

6. Compute agent_alerts (Warning + Critical) and POST to n8n (fire-and-forget).

7. Run the dwell-rule auto-maintenance: if a machine has been Critical for ≥ 10 s and is not in cooldown, drain any active what-if scenario (or run a standard reset), then emit STARTING + SUCCESS records.

8. Compute factory-level analytics (plant health score, most unstable machine, count needing attention).

9. Merge in-process agent_actions with the last 10 of RECENT_AGENT_ACTIONS (n8n-driven) and ship as the WS payload.

### Startup hooks

- _create_tables — calls Base.metadata.create_all(engine) and runs a tiny SQLite ALTER-TABLE migration to add the is_whatif column to a pre-existing maintenance_logs table (since create_all cannot alter).

- _warmup_llm_on_start — daemon thread that calls warmup_llm() so the first chatbot LLM query isn't extra-slow.

### UTF-8 stdout shim

The first 12 lines of main.py reconfigure sys.stdout and sys.stderr to UTF-8 with errors="replace" because Windows + Python 3.14 defaults to cp1252, and the codebase has many emoji print() calls that would otherwise propagate UnicodeEncodeError up through request handlers as 500s.

## 7. Closed-Loop n8n Agent Pipeline

### Workflow shape (n8n/workflows/synaptic_fabric_agent.json)

[Webhook /synfab-alert]

│

▼

[Normalize Alert]    ← strips machine_id / severity / risk into a flat shape

│

▼

[Switch on severity]

│ ──── CRITICAL ──── ▶ [Ollama: Justify (phi3, cite root cause)]

│                          │

│                          ▼

│                   [POST /agent/action]  → AUTO_MAINTENANCE

│

└──── WARNING ──── ▶ [POST /agent/action]  → NOTIFY_OPERATOR

(skip LLM)

### Outbound from FastAPI

backend_fastapi/app/n8n_client.py — send_alert(machine, severity, risk):

- Fire-and-forget httpx.AsyncClient POST to http://localhost:5678/webhook/synfab-alert.

- Throttled per machine (won't spam n8n if a machine sits in Critical for many ticks).

- Swallows connection errors — if n8n is down, the WS loop keeps running.

### Inbound to FastAPI

backend_fastapi/app/agent_webhook.py — POST /agent/action accepts:

class AgentAction(BaseModel):

machine_id: str

action: str                    # "AUTO_MAINTENANCE" | "NOTIFY_OPERATOR"

reason: Optional[str]

justification: Optional[Any]   # passthrough from n8n's Ollama node

source: Optional[str] = "n8n"

For AUTO_MAINTENANCE, the handler:

1. Checks if any what-if snapshot is active and either drains the entire scenario (logging WHAT_IF_REVERT for every restored machine) or applies the standard reset (tool_wear *= 0.15, vibration *= 0.25, temperature -= 8, torque *= 0.92).

2. Resets inbound propagation buffers.

3. Appends a record to in-memory RECENT_AGENT_ACTIONS (capped at 50).

4. Persists a MaintenanceLog row with source="n8n" (or whatever n8n reports), capturing the machine's health_status and prediction at the moment of the action.

### Failure modes

- Ollama not pulled → CRITICAL branch errors at the Ollama HTTP node; WARNING branch and corrective POST still work. Pull once with docker exec synfab_ollama ollama pull phi3.

- n8n down → n8n_client.py swallows; nothing else breaks.

- /agent/action 500 → in-process auto-maintenance still fires via the dwell rule (10 s in Critical), so the system is corrective even with n8n disconnected.

## 8. Local Two-Tier Chatbot

### Routing

The dashboard chatbot's /chat/stream endpoint is a deliberately n8n-free path. It runs entirely in-process for two reasons:

- Latency. Routing through an n8n HTTP-hop on every keystroke would push median latency above 500 ms.

- Determinism. Most operator queries are metric lookups (temperature of M_3, which is worst), which deserve a fast-path lookup, not an LLM round-trip.

user query

│

▼

[fast deterministic path]   ←── regex match against LIVE_MACHINES

│  metric lookups, status, comparisons, plant health, alerts

│  ~10–300 ms, NO LLM call

▼

[LLM reasoning path]        ←── triggered only when reasoning words appear

(why / explain / how / should / cause / suggest / trend / history)

- slim prompt (focused machine; history added only when explicitly asked)

- Ollama params: num_predict=80, num_ctx=1024, num_thread=8

- streamed back as text/plain, token by token (typing effect)

### Implementation

| Concern | File |
| --- | --- |
| Endpoint + tier router | `backend_fastapi/app/chatbot_api.py` |
| Ollama HTTP client | `backend_fastapi/chatbot/llm_client.py` |
| RAG service (Qdrant) | `backend_fastapi/chatbot/rag_service.py` |
| History tools | backend_fastapi/chatbot/history_tools.py |
| Streaming consumer + Stop button | `frontend_dashboard/src/components/Chatbot.js` |

The legacy /chat endpoint is preserved as a non-streaming fallback that shares the same fast/LLM routing.

## 9. Maintenance Logging & Audit Trail

### Schema

class MaintenanceLog(Base):

__tablename__ = "maintenance_logs"

id            = Column(Integer, primary_key=True)

timestamp     = Column(DateTime, default=datetime.utcnow, index=True)

machine_id    = Column(String, index=True)

action        = Column(String)   # MANUAL | AUTO | WHAT_IF_SPIKE | WHAT_IF_REVERT

status        = Column(String)   # STARTING | SUCCESS | APPLIED | IGNORED

source        = Column(String)   # dashboard | dwell_rule | n8n | dashboard_whatif

reason        = Column(Text, nullable=True)

health_status = Column(String, nullable=True)

risk          = Column(Float, nullable=True)

is_whatif     = Column(Boolean, default=False, nullable=False)

### Persistence sites

- manual_maintenance (dashboard button) — one APPLIED row + STARTING/SUCCESS WS records.

- Dwell-rule auto-maintenance — STARTING + SUCCESS rows.

- agent_action from n8n — one APPLIED row.

- inject_spike — one WHAT_IF_SPIKE row per affected machine (origin + chain neighbours).

- Any maintenance during an active what-if — one WHAT_IF_REVERT row per restored machine.

### Endpoints

| Method · Path | Returns | Filters |
| --- | --- | --- |
| GET /maintenance/logs | Newest-first JSON list | minutes, machine_id, limit (≤ 10000) |
| GET /maintenance/logs/download | File download | `format=csv\ |

CSV columns: id, timestamp_utc, machine_id, action, status, source, is_whatif, health_status, risk, reason.

### Path-conflict gotcha

/maintenance/logs clashes with POST /maintenance/{machine_id} — FastAPI matches /maintenance/logs against {machine_id}=logs and returns 405 if methods differ. The download endpoint at /maintenance/logs/download has an extra path segment, so it doesn't conflict. The list endpoint avoids the conflict because it's GET while the manual-maintenance route is POST with a single segment under it — FastAPI's method-based routing disambiguates here, but if a GET /maintenance/{machine_id} were ever added it would shadow the list endpoint.

## 10. What-If Spike Scenarios

Operators can hypothetically perturb any machine's state to visualize blast-radius effects, fully reversible by any subsequent maintenance action.

### Endpoint

POST /spike/{machine_id}

Content-Type: application/json

{ "temperature": 318, "vibration_index": 0.95,

"tool_wear": 0.92, "torque": null, "note": "..." }

### Mechanism

1. Compute delta for each provided field, relative to the current state of the origin machine.

2. For every machine in the chain (M_1–M_4), look up an attenuation factor by signed chain-distance:

| Distance | -3 | -2 | -1 | 0 (origin) | +1 | +2 | +3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Attenuation | 0.05 | 0.10 | 0.20 | 1.00 | 0.60 | 0.40 | 0.25 |

3. Snapshot each machine's current state into state.WHATIF_SNAPSHOTS[mid] (only if not already snapshotted — preserves the original baseline if multiple sequential spikes occur).

4. Apply delta * attenuation to each machine's chosen fields, with per-field clamping.

5. Record one WHAT_IF_SPIKE log row per affected machine, with is_whatif=True.

6. Return the affected list:

``json    {      "status": "success",      "origin": "M_2",      "affected": [        {"machine_id":"M_1","attenuation":0.2,"is_origin":false,"applied":{...}},        {"machine_id":"M_2","attenuation":1.0,"is_origin":true,"applied":{...}},        {"machine_id":"M_3","attenuation":0.6,"is_origin":false,"applied":{...}},        {"machine_id":"M_4","attenuation":0.4,"is_origin":false,"applied":{...}}      ],      "hint": "Performing maintenance on ANY machine in this scenario will revert the whole plant to its pre-spike state."    }    ``

### Empirical trace (spike M_2 with vibration=0.95, tool_wear=0.92)

| Machine | Attenuation | Δ vib | Δ wear | Δ temp | Health after spike |
| --- | --- | --- | --- | --- | --- |
| M_1 | 0.2 | +0.090 | +0.167 | -0.03 | Healthy (mild reflection) |
| M_2 | 1.0 | +0.324 | +0.812 | +2.21 | Critical (origin) |
| M_3 | 0.6 | +0.554 | +0.495 | +2.20 | Critical (chain effect) |
| M_4 | 0.4 | +0.184 | +0.327 | +0.37 | Healthy |

A single failure visibly drags M_3 into Critical and adds measurable load to M_1 and M_4 — exactly the failure-cascade visualization operators need.

### Group revert

Maintenance on any machine in an active scenario triggers a full revert:

{ "status":"success", "machine_id":"M_2", "reverted_whatif":true,

"co_reverted":["M_1","M_3","M_4"] }

All four maintenance paths share the same drain-the-scenario logic:

1. Manual dashboard button.

2. Dwell-rule auto-maintenance.

3. n8n POST /agent/action.

4. (Implicit) — even if the maintained machine isn't the spike origin, the entire active scenario is restored.

## 11. Auto-Maintenance Logic

### Dwell rule

CRITICAL_DWELL_SECONDS = 10

MACHINE_CRITICAL_SINCE = {}      # machine_id → wall-clock time entered Critical

MAINTENANCE_COOLDOWN  = {}       # machine_id → time of last reset

COOLDOWN_TIME = 20

A machine triggers auto-maintenance when all three hold:

1. health_status == "Critical" for the current tick.

2. It has been continuously Critical for ≥ CRITICAL_DWELL_SECONDS (10 s).

3. It is not within the COOLDOWN_TIME window (20 s) since its last reset.

Why a dwell rule (instead of a metric threshold)? Because real-world high-risk machines often have individual signals (e.g. tool_wear) that stay below their physics-override thresholds, but the fused prediction stays in Critical territory. Dwell-time gives a uniform "this machine has been bad long enough — service it" criterion that doesn't depend on which signal is the loudest right now.

### Effects

| Trigger | What happens to MACHINE_MEMORY |
| --- | --- |
| Manual maintenance (button) | tool_wear *= 0.15, vibration *= 0.25, temperature -= 8 (floor 295), torque *= 0.92 |
| Dwell-rule auto-maintenance | tool_wear *= 0.15, vibration *= 0.25, temperature -= 10, torque *= 0.9 (slightly stronger than manual) |
| n8n AUTO_MAINTENANCE callback | Same as manual (* 0.15, * 0.25, -8, * 0.92) |
| Any of the above during what-if | Snapshot from WHATIF_SNAPSHOTS[mid] is restored verbatim |

In all cases reset_inbound_buffers(mid) is called to flush stale upstream queue entries.

### WS popup contract

The frontend useMachineStream hook watches the agent_actions field of the WS payload and de-duplicates by ${machine_id}-${timestamp}-${status}-${action}. The popup queue serializes messages with a 3-second display each:

| Action | STARTING | SUCCESS |
| --- | --- | --- |
| AUTO_MAINTENANCE | ⚙️ Performing maintenance on M_x | ✅ Maintenance completed for M_x |
| MANUAL_MAINTENANCE | same | same |
| WHAT_IF_REVERT | ↩️ Reverting what-if scenario on M_x | ✅ M_x restored to pre-spike state |

(Emoji glyphs are intentional in the popup queue messages — the user-facing text stays minimal but distinguishable.)

## 12. Frontend Dashboard

### Tech stack

- React via create-react-app.

- @react-three/fiber + drei for the 3D digital twin.

- recharts for the temperature line chart.

- Inter (UI text) + JetBrains Mono (numerical metrics) loaded via Google Fonts in public/index.html.

### Layout (top to bottom)

1. Header — eyebrow SYNAPTIC FABRIC, title Mission Control, subtitle line; bottom border.

2. 3D Digital Twin — four named machine geometries on a chain-shaped plinth.

3. Machine Card grid — one card per machine.

4. Factory Intelligence — plant health %, most unstable machine, total machines, needs-attention list. Each as a flat stat card with a monospaced numeric value.

5. Agent Alerts — borderless rows with thin colored left-edge accents (amber for Warning, red for Critical) and a tinted background.

6. Recent Maintenance Activity (collapsible, closed by default) — auto-refreshes every 5 s via GET /maintenance/logs?minutes=10&limit=200. Sticky-header table with max-height: 220px, vertical scrollbar, what-if rows tinted amber with a WHAT-IF badge.

7. Maintenance Log Downloads (collapsible) — CSV / JSON export buttons.

8. Live Temperature (collapsible) — recharts line chart with a 1/5/10-min window selector.

9. AI Assistant — fixed bottom-right pill button; opens a 340 × 440 chat popup.

10. Auto-maintenance popup — fixed top-center 3-second toast queue.

### Machine Card composition

┌────────────────────────────────────────────────────┐

│ M_3                                  [HEALTHY · ●] │

│ CNC Milling Tool                                   │

│ Surface finishing                                  │

├────────────────────────────────────────────────────┤

│ ┌──────────────────┬──────────────────┐            │

│ │ TEMP             │ TORQUE           │            │

│ │ 296.06 K         │ 44.18 Nm         │            │

│ ├──────────────────┼──────────────────┤            │

│ │ WEAR             │ VIBRATION        │            │

│ │ 10.2%            │ 0.153            │            │

│ └──────────────────┴──────────────────┘            │

│                                                    │

│ FAILURE RISK                            12%        │

│ ▓▓░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░         │

│                                                    │

│ ┌──────────────────┬──────────────────┐            │

│ │ RUL CYCLES       │ EST. TIME        │            │

│ │ 280              │ 5.6 hrs          │            │

│ └──────────────────┴──────────────────┘            │

│                                                    │

│ [   Perform Maintenance   ]                        │

│ [    Inject Spike (◊)     ]                        │

│                                                    │

│ INSPECTION                                         │

│ No issues detected.                                │

└────────────────────────────────────────────────────┘

### useMachineStream hook

`frontend_dashboard/src/useMachineStream.js` handles:

- WebSocket connection to ws://127.0.0.1:8000/ws/machines.

- Auto-reconnect with 2 s backoff on close.

- A popup queue with de-duplication (shownActionsRef.current Set).

- Mapping the raw WS payload into the shape the rest of the app expects.

### Design system

- Single-color background #050810 (no radial gradient) so cards read as foreground.

- Stat values use JetBrains Mono with font-feature-settings: "tnum", "zero" for tabular alignment and unambiguous zeros.

- Thin 1 px borders (rgba(148, 163, 184, 0.10)) instead of heavy box-shadows.

- Status pills are dot-prefixed labels with thin matching borders — Healthy = emerald (#34d399), Warning = amber (#fbbf24), Critical = coral (#f87171).

- Custom thin scrollbars (*::-webkit-scrollbar { width: 8px }) globally, using rgba(148, 163, 184, 0.25) on the thumb.

- All emojis stripped from user-facing surfaces (replaced with typographic emphasis: weight, color, letter-spacing); emojis remain only inside the auto-maintenance popup queue text strings.

## 13. Data Model & Persistence

### Engine

SQLite at ./machine_data.db with check_same_thread=False for FastAPI's threaded request handling.

### Tables

| Table | Purpose | Lifetime |
| --- | --- | --- |
| machine_logs | Per-tick metric snapshot of every machine | Cleaned every 60 s — rows older than 30 minutes are deleted |
| maintenance_logs | Every maintenance / spike / revert event | Permanent (until explicit purge) |

### Migrations

Alembic is not wired in (intentionally — this is a demo platform, not a production migration story). Instead:

1. Base.metadata.create_all(engine) runs on FastAPI startup to create any missing tables.

2. A bespoke SQLite ALTER-TABLE migration runs immediately after create_all to add the is_whatif column to a pre-existing maintenance_logs table (because create_all cannot alter existing tables and the column was added later).

Future schema changes that aren't pure additions would need either a manual DROP/recreate or an Alembic adoption.

### Why SQLite (not PostgreSQL)

The architecture brief mentions PostgreSQL but this build uses SQLite because:

- Zero infrastructure footprint matters for a local-only demo.

- Write-rate (~4 rows/sec to machine_logs, plus occasional maintenance_logs writes) is well under SQLite's ceiling.

- The cleanup job keeps machine_logs capped at ~30 minutes ≈ 7,200 rows, which SQLite handles without breathing hard.

## 14. Deployment & Local Setup

### Prerequisites

- Windows / macOS / Linux with Docker Desktop or compatible

- Python 3.11+ (this build runs on 3.14 with the UTF-8 stdout shim)

- Node 18+

### Startup sequence

# 1. Local infrastructure

docker compose up -d n8n qdrant ollama

docker exec -it synfab_ollama ollama pull phi3        # CRITICAL branch

docker exec -it synfab_ollama ollama pull phi3:mini   # dashboard chat

# 2. Backend (host)

./venv/Scripts/python.exe -m uvicorn backend_fastapi.app.main:app \

--host 0.0.0.0 --port 8000

# 3. Frontend (host)

cd frontend_dashboard && npm start

# 4. Activate the n8n workflow (one-time, in the n8n UI)

#    Open http://localhost:5678 → Create owner account → Import

#    n8n/workflows/synaptic_fabric_agent.json → Activate.

### Default ports

| Service | URL |
| --- | --- |
| FastAPI | http://localhost:8000 |
| React dashboard | http://localhost:3000 |
| n8n | http://localhost:5678 |
| Qdrant | http://localhost:6333 |
| Ollama | http://localhost:11434 |

### Smoke-test commands

# Manual maintenance on M_3

curl -X POST http://localhost:8000/maintenance/M_3

# Inject a what-if spike with chain propagation

curl -X POST http://localhost:8000/spike/M_2 \

-H "Content-Type: application/json" \

-d '{"vibration_index":0.95,"tool_wear":0.92}'

# Trigger n8n via its production webhook

curl -X POST http://localhost:5678/webhook/synfab-alert \

-H "Content-Type: application/json" \

-d '{"machine_id":"M_3","severity":"CRITICAL","prediction":0.92}'

# Pull the last 10 minutes of maintenance log

curl 'http://localhost:8000/maintenance/logs?minutes=10' | jq .

## 15. Operational Gotchas

### Windows + Python 3.14: UTF-8 stdout

The codebase has many emoji print() calls that crash on cp1252. The fix lives at the very top of main.py:

for _stream in (sys.stdout, sys.stderr):

try:

_stream.reconfigure(encoding="utf-8", errors="replace")

except Exception:

pass

If you ever add a new entrypoint, replicate this block — otherwise the first emoji print() will trip UnicodeEncodeError and surface as a 500 from a request handler.

### Docker Desktop's path-with-space bug

If your Windows username contains a space (e.g. SIDDHI KARHEKAR), Docker Desktop's Inference Manager and Secrets Engine will fail to bind their Unix sockets at %LOCALAPPDATA%\Docker\run\dockerInference and %LOCALAPPDATA%\docker-secrets-engine\engine.sock because unix:// URLs split on the space. Symptom: a startup dialog with "An unexpected error occurred · Docker Desktop encountered an unexpected error and needs to close · The filename, directory name, or volume label syntax is incorrect."

Workaround that worked:

1. Kill all Docker processes: taskkill /F /IM "Docker Desktop.exe" and taskkill /F /IM "com.docker.backend.exe".

2. Shut down WSL: wsl --shutdown.

3. Rename the broken IPC dirs (the zombie pipe handles can't be deleted, only renamed) using Python's os.rename — Windows' cmd /c ren and rmdir both fail with ERROR_INVALID_NAME (123):

``python    import os, time    ts = int(time.time())    for src in [        r'C:\Users\NAME WITH SPACE\AppData\Local\Docker\run',        r'C:\Users\NAME WITH SPACE\AppData\Local\docker-secrets-engine',    ]:        os.rename(src, f"{src}_broken_{ts}")    ``

4. Relaunch Docker Desktop. It creates fresh run/ and docker-secrets-engine/ directories and starts cleanly.

The renamed *_broken_* directories are harmless on disk; they release after a Windows reboot.

### Soft-failure modes

- n8n down → n8n_client.py swallows; WS keeps streaming.

- Ollama not pulled → CRITICAL n8n branch errors at the Ollama node; dashboard chatbot drops to fast deterministic path; everything else keeps working.

- gnn.pth corrupt or missing → gnn_inference.py no longer loads it at all; the chain-aware physics fallback is the canonical path.

- WebSocket disconnect → frontend's useMachineStream reconnects with 2 s backoff.

## 16. Endpoint Reference

| Method | Path | Purpose |
| --- | --- | --- |
| WS | /ws/machines | Live stream — machines, analytics, alerts, actions (~1 Hz) |
| GET | /history?minutes=N | Per-machine temperature history (1 ≤ N ≤ 60) |
| POST | /maintenance/{machine_id} | Manual maintenance — drains active what-if scenario or runs standard reset |
| POST | /spike/{machine_id} | Inject a what-if spike with chain propagation |
| POST | /agent/action | Closed-loop hook called by n8n |
| GET | /agent/actions?limit=N | In-memory recent agent actions (n8n-driven) |
| GET | /agent/health | { "status": "ok", "live_machines": [...] } |
| GET | /maintenance/logs | DB-backed list with minutes, machine_id, limit filters |
| GET | /maintenance/logs/download | CSV or JSON file download |
| POST | /chat | Two-tier chat (non-streaming) |
| POST | /chat/stream | Two-tier chat (text/plain streamed) |

## 17. WebSocket Payload Reference

{

"machines": [

{

"machine_id": "M_3",

"temperature": 296.06,

"torque": 44.18,

"tool_wear": 0.1024,

"vibration_index": 0.1531,

"pinn_temperature": 275.485,    // PINN heat-eq prediction

"pinn_risk": 0.0,

"gnn_risk": 0.21,               // chain-aware physics fallback

"anomaly_score": 0.063,

"anomaly_ml_score": 0.05,

"failure_probability": 0.121,

"prediction": 0.121,            // fused final risk

"rul_cycles": 280,

"rul_time": "5.6 hrs",

"future_temperature": 297.31,

"health_status": "Healthy",

"root_cause": [...],            // analyzer rule output

"alerts": [

{ "level": "WARNING", "message": "High temperature" }

],

"shap": { "tool_wear": 0.31, "temperature": 0.18, ... },

"ai_explanation": "Healthy | anomaly=0.063 | risk=0.121 | gnn=0.21",

"ai_reason": "Primary factor: tool_wear"

}

/* …M_1..M_4 */

],

"factory_analytics": {

"plant_health_score": 88.2,

"most_unstable_machine": "M_4",

"total_machines": 4,

"machines_needing_attention": []

},

"agent_alerts":  [

{ "machine_id": "M_3", "level": "CRITICAL", "message": "Failure risk 92%" }

],

"agent_actions": [

{ "machine_id": "M_3", "action": "AUTO_MAINTENANCE",

"status": "STARTING", "timestamp": 1778064686.1494906 },

{ "machine_id": "M_3", "action": "AUTO_MAINTENANCE",

"status": "SUCCESS",  "timestamp": 1778064686.1504905 }

]

}

## 18. Tuning History (What Changed and Why)

This section documents the most consequential tuning decisions made during development, so future contributors don't undo them inadvertently.

### M_3-stuck-in-Critical fix

Symptom: M_3 (CNC Milling Tool) was perpetually in Warning/Critical, snapping back into Critical seconds after any maintenance.

Root causes (all real, all fixed):

1. GNN saturation. The pretrained gnn.pth produced near-constant 1.0 logits → 0.15 floor on every fused prediction. → Replaced with chain-aware physics fallback (see §4.2).

2. Simulator over-coupling. M_3 vibration update was V_new = 0.98·V_old + 0.6·delayed_vib, steady-state amplification 0.6/(1 - 0.98) = 30×. → Tightened to 0.94·V_old + 0.18·delayed_vib, steady-state 3×. Heat injection from upstream vib * 0.4 → * 0.15. Wear coupling * 0.0025 → * 0.0010.

3. M_2 monotonic drift. M_2 vibration update had no decay term, so it climbed without bound → pinned M_3. → Added * 0.985 self-decay.

4. Stale propagation buffers. Maintenance reset state but M_2→M_3 queue still held the pre-maintenance high values → M_3 re-entered Critical within 3 ticks. → Added reset_inbound_buffers(mid) to all maintenance paths.

5. GNN weighting. Original adjacency had adj[i+1][i] = 1.0 (upstream) and adj[i][i] = 1.0 (self) → after row normalization, M_3 weighted upstream equally with itself. → Reduced upstream edge to 0.5, reverse to 0.15.

### Empirical verification (post-fix)

| Phase | M_3 Critical ticks | M_3 max vib | M_3 max pred |
| --- | --- | --- | --- |
| 60 s warm-up (no maintenance) | 0 | 0.738 | 0.700 |
| 30 s after M_3 maintenance | 0 | 0.134 | 0.216 |
| 30 s after maintenance, with M_2 spiking to vib=0.915 | 0 | 0.506 | 0.205 |

### Maintenance popup wiring

Symptom: Clicking "Perform Maintenance" on a card showed no popup.

Cause: Manual maintenance only wrote to the DB, never to agent_actions (which the WS payload exposes and the popup hook watches).

Fix: manual_maintenance now appends STARTING + SUCCESS records into RECENT_AGENT_ACTIONS (timestamp offset by +0.001 s to keep them distinct in the dedup set). The frontend popup matcher was simultaneously broadened from a single-action match (AUTO_MAINTENANCE) to a tuple matcher covering AUTO_MAINTENANCE, MANUAL_MAINTENANCE, and WHAT_IF_REVERT.

### Schema compatibility

When the is_whatif column was added, the pre-existing maintenance_logs table broke INSERTs with OperationalError: table maintenance_logs has no column named is_whatif. Fix: lightweight ALTER-TABLE migration on FastAPI startup that introspects PRAGMA table_info(maintenance_logs) and adds the column only if missing.

## 19. Repository Structure

SynapticFabricfinal/

├── backend_fastapi/

│   ├── ai_engine/

│   │   ├── machine_analyzer.py     # GNN + PINN + ML + anomaly fusion + alerts + RUL

│   │   └── root_cause.py           # Per-machine root-cause rules

│   ├── analytics/

│   │   └── factory_analytics.py    # Plant-level metrics

│   ├── app/

│   │   ├── main.py                 # FastAPI entrypoint, WS, /history, /maintenance, /spike

│   │   ├── agent_webhook.py        # /agent/action, /agent/actions, /agent/health,

│   │   │                           # /maintenance/logs, /maintenance/logs/download

│   │   ├── chatbot_api.py          # /chat + /chat/stream (two-tier router)

│   │   ├── n8n_client.py           # Outbound alerts to n8n (throttled, fire-and-forget)

│   │   └── state.py                # LIVE_MACHINES + WHATIF_SNAPSHOTS

│   ├── chatbot/

│   │   ├── llm_client.py           # Ollama HTTP client + warmup

│   │   ├── rag_service.py          # Qdrant retrieval

│   │   └── history_tools.py        # Conversation memory

│   └── database/

│       ├── database.py             # SQLAlchemy engine + session

│       ├── models.py               # MachineLog, MaintenanceLog

│       └── maintenance_log.py      # record_maintenance() helper

│

├── digital_twin/

│   └── simulator.py                # 4-node causal-physics simulator (M_1..M_4)

│

├── ml_models/

│   ├── graph_builder.py            # Chain-encoded adjacency

│   ├── gnn_model.py                # SimpleGNN (Linear over message pass)

│   ├── gnn_inference.py            # Chain-aware physics fallback (replaces saturating ckpt)

│   ├── failure_model.py            # ML failure probability

│   ├── anomaly_model.py            # ML anomaly score

│   ├── transformer_inference.py    # Short-horizon temperature forecast

│   └── explainer.py                # SHAP attributions

│

├── pinn_model/

│   ├── heat_pinn.py                # Physics-informed network (heat-eq residual loss)

│   └── pinn_inference.py           # predict_temp(temp, torque, air)

│

├── frontend_dashboard/

│   ├── public/index.html           # Inter + JetBrains Mono fonts, custom scrollbars

│   └── src/

│       ├── App.js                  # Mission Control dashboard

│       ├── useMachineStream.js     # WS client + popup queue

│       ├── components/

│       │   ├── MachineCard.js / .css   # Industrial card with 2x2 metric grid + spike form

│       │   ├── FactoryTwin3D.js        # 3D twin

│       │   ├── TemperatureChart.js     # recharts line chart

│       │   ├── AlertPanel.js

│       │   └── Chatbot.js

│       └── utils/machineInfo.js    # name + process for M_1..M_4

│

├── n8n/

│   ├── workflows/synaptic_fabric_agent.json

│   └── README.md

│

├── docker-compose.yml              # n8n + Qdrant + Ollama

├── machine_data.db                 # local SQLite (gitignored)

├── CLAUDE.md                       # Repo-internal architecture notes

└── PROJECT_REPORT.md               # this file

## 20. Future Work & Known Limitations

### Future work

- Persist WHATIF_SNAPSHOTS across restarts. Today they live only in process memory — a backend restart during an active scenario means snapshots are lost and machines can't be reverted to original. A small SQLite table keyed on machine_id would solve this.

- Retrain the GNN. The current chain-aware fallback is physics-correct but linear. A properly normalized + retrained gnn.pth (with feature standardization in graph_builder.py) could capture nonlinear chain interactions the fallback misses.

- Alembic migrations. Replace the bespoke ALTER-TABLE startup hook with a real migration story before adding any non-additive schema changes.

- PostgreSQL adapter. Wire it in (it's mentioned in the architecture brief). The DB access is already through SQLAlchemy Session, so the engine swap is configuration.

- Production-grade n8n auth. The current setup uses a single owner account; for any multi-user deployment the workflow should run under an API-key-gated execution context.

- Per-tenant config. All thresholds (CRITICAL_DWELL_SECONDS, COOLDOWN_TIME, ALERT_THRESHOLD, fusion weights) are hardcoded in main.py and machine_analyzer.py. A YAML config + reload endpoint would make this tunable without redeploys.

### Known limitations

- Single-process simulator. MACHINE_MEMORY and WHATIF_SNAPSHOTS are module-level state. Horizontal scaling would need an external state store (Redis would be the natural fit).

- No plant-wide spike test scenarios. What-ifs are anchored on a single origin machine; multi-origin scenarios (e.g. simultaneous M_1 and M_3 failure) require sequential spikes which compound deltas in a way that may not match a true coincident-failure profile.

- Fixed chain topology. The simulator and GNN assume a 4-node linear chain. A branching or recirculating topology would need rewrites in simulator.py, graph_builder.py, and _chain_targets() in main.py.

- No authentication. The dashboard, FastAPI endpoints, and n8n owner account are all open on localhost — fine for local demo, not for any networked deployment.

Last updated 2026-05-06 by the development team.